COMPANY NAME / LOGO

INSPECTION REGISTER:  FALL ARREST SYSTEM

NOTE: Indicate yes/no to all applicable questions

|  | DATE |  | DATE |  | DATE |  | DATE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | YES | NO | YES | NO | YES | NO | YES | NO |
| Anchorage points |  |  |  |  |  |  |  |  |
| 1. Do workers know appropriate anchorage points for each task that requires a fall-arrest or restraint system? |  |  |  |  |  |  |  |  |
| 2. Are all anchorage points stable, substantial, and have sufficient strength to withstand twice the potential impact energy of the free-fall? |  |  |  |  |  |  |  |  |
| 3. Are all anchorage points for body harness located at shoulder height? Are anchorage points for self-retracting lifeline located overhead |  |  |  |  |  |  |  |  |
| 4. Can a worker move from one station to another or climb up and down without exposure to a fall? |  |  |  |  |  |  |  |  |
| 5. If the lifeline, lanyard, or self-retracting lifeline is not permanently attached to an anchorage point at the elevated work area, is the first worker up or the last worker down protected while climbing and traversing? |  |  |  |  |  |  |  |  |
| Vertical Lifelines | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Does the lifeline have a minimum breaking strength of 5,000 pounds? (2-258 kg) |  |  |  |  |  |  |  |  |
| 2. Is the lifeline protected from abrasive or cutting edges? |  |  |  |  |  |  |  |  |
| 3. Does the system provide fall protection as the worker connects to and releases from the lifeline? |  |  |  |  |  |  |  |  |
| 4. Is the lifeline arranged so workers never have to hold it for balance? (A lifeline should never be used for balance) |  |  |  |  |  |  |  |  |
| 5. Is the vertical segment integrated with horizontal segment to provide continuous fall protection? |  |  |  |  |  |  |  |  |
| Fall arresters | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Is the fall arrester compatible with the lifeline on which it is to be installed or operated? |  |  |  |  |  |  |  |  |
| 2. Is the fall arrester in operational condition? |  |  |  |  |  |  |  |  |
| 3. Is the fall arrester equipped with changeover lever that allows it to become a stationary anchor on the lifeline? |  |  |  |  |  |  |  |  |
| 4. Is the fall arrester equipment with locking mechanism that prevents unintentional opening of the device and subsequent disengagement from the lifeline? |  |  |  |  |  |  |  |  |
| 5. Is the fall arrester’s “up’ direction marked properly so that the equipment can be attached to the line correctly? |  |  |  |  |  |  |  |  |
| 6. Is the fall arrester included in a regular maintenance program? |  |  |  |  |  |  |  |  |
| Horizontal lifeline | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Has the entire horizontal lifeline system been designed and approved by a competent person |  |  |  |  |  |  |  |  |
| 2. Have the anchorages to which the lifeline is attached been designed and specifically for a horizontal lifeline? |  |  |  |  |  |  |  |  |
| 3. Has the designer of the system approved the number of workers who will be using it? |  |  |  |  |  |  |  |  |
| 4. Is the rope or cable free from signs of wear or abrasion? |  |  |  |  |  |  |  |  |
| 5. Does the rope or cable have the required initial sag? |  |  |  |  |  |  |  |  |
| 6. Have the workers been warned about potential falls? Have the clearances been checked? |  |  |  |  |  |  |  |  |
| 7. Is the hardware riding on the horizontal lifeline made of steel? (Aluminium is not permitted because it wears excessively) |  |  |  |  |  |  |  |  |
| Lanyards | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Is the lanyard length as short as necessary and in no case greater than 6 feet? (1,8 m) |  |  |  |  |  |  |  |  |
| 2. Are manually adjustable lanyards used when it is desirable to be able to take slack out of the lanyard? |  |  |  |  |  |  |  |  |
| Lanyards | YES | NO | YES | NO | YES | NO | YES | NO |
| 3. Does the lanyard have a shock – absorbing to limit the arresting forces to 500-600 pounds? (227-272 kilograms) |  |  |  |  |  |  |  |  |
| 4. If the lanyard has a shock absorber, is it obvious to the user that the shock absorber has been deployed? (Is there a warning label, broken pouch, etc.) |  |  |  |  |  |  |  |  |
| 5. Have you prohibited tying of knots from the lanyard to the lifeline? (Mechanical rope grabs or fall arresters must be used) |  |  |  |  |  |  |  |  |
| Snap Hooks | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Have double-locking snap hooks been used? |  |  |  |  |  |  |  |  |
| 2. Is the snap hook attached to the D-ring, eyebolt, or other hardware in a manner approved by the manufacturer of snap hook? |  |  |  |  |  |  |  |  |
| 3. Are snap hooks inspected regularly for stress, wear, distortion, and spring failure? |  |  |  |  |  |  |  |  |
| 4. Are snap hooks arranged so they are never connected to each other? (They should NOT be connected to each other) |  |  |  |  |  |  |  |  |
| Body Harnesses | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Are full – body harnesses selected for a particular job equipped with all necessary attachment points (for fall arresting, work positioning, descent control, rescue, or ladder fall – protection systems |  |  |  |  |  |  |  |  |
| 2. Are body harnesses inspected regularly for wear, abrasion, broken stitching, and Missing hardware |  |  |  |  |  |  |  |  |
| 3. Is the Velcro type of closure prohibited from all load-bearing connections? |  |  |  |  |  |  |  |  |
| 4. Have workers been instructed in the use and care of body harnesses/body Belts? |  |  |  |  |  |  |  |  |
| Arrest systems checklist | YES | NO | YES | NO | YES | NO | YES | NO |
| 1. Has the free-fall distances been considered, so that a worker will not strike a lower surface or object before the fell is arrested? |  |  |  |  |  |  |  |  |
| 2. Have pendulum-swing fall hazards been eliminated? |  |  |  |  |  |  |  |  |
| 3. Have methods to retrieve fallen workers been planned |  |  |  |  |  |  |  |  |
| 4. Is all fall-arrest equipment free of potential damage from welding, chemical, or sandblasts? |  |  |  |  |  |  |  |  |
| 5. Are all components of the system compatible according to the manufacture’s instruction? |  |  |  |  |  |  |  |  |
| 6. Have employees been properly trained in the following issues? |  |  |  |  |  |  |  |  |
| • Manufacturer’s recommendations, restrictions, instructions, and warning • Location of appropriate anchorage points and attachment techniques • Problems associated with elongation, deceleration distance, method of use, inspection, and storage |  |  |  |  |  |  |  |  |
| 7. Are all regular inspections performed by trained inspectors? |  |  |  |  |  |  |  |  |
| 8. Are written reports maintained? |  |  |  |  |  |  |  |  |
| Inspection Details |  |  |  |  |  |  |  |  |
| Inspector Name |  |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |
| Responsible Person |  |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |